EXPERIMENT 3
STEPPER MOTOR USING AT89C51 USING PROTEUS

AIM: To write an assembly language program for Stepper Motor using AT89C51 using Keil and Proteus.

SOFTWARE REQUIRED: Proteus 8 software

ALGORITHM

Load the first step pattern 09H on Port 2.

Apply a delay.

Load 0CH, 06H and 03H successively with delays.

Repeat the sequence continuously to rotate the stepper motor.

PROGRAM

ORG 0000H
UP: MOV P2,#09H
    ACALL DELAY
    MOV P2,#0CH
    ACALL DELAY
    MOV P2,#06H
    ACALL DELAY
    MOV P2,#03H
    ACALL DELAY
    SJMP UP

DELAY: MOV R4,#18
H1:    MOV R3,#255
H2:    DJNZ R3,H2
       DJNZ R4,H1
       RET
END

OUTPUT IMAGE

RESULT

Thus the program has been successfully verified and executed.