EXPERIMENT NO. 25

AIM

To model the Producer-Consumer problem using SystemC.

ALGORITHM

Create a producer and a consumer module.

Connect them using an sc_fifo channel.

Producer generates values from 1 to 5.

Consumer reads the values from the FIFO.

Display produced and consumed values.

PROGRAM

#include <systemc.h> SC_MODULE(Producer) { sc_fifo_out<int> out; void produce() { for(int i = 1; i <= 5; i++) { cout << "Produced: " << i << endl; out.write(i); wait(1, SC_SEC); } } SC_CTOR(Producer) { SC_THREAD(produce); } }; SC_MODULE(Consumer) { sc_fifo_in<int> in; void consume() { int value; for(int i = 1; i <= 5; i++) { in.read(value); cout << "Consumed: " << value << endl; } } SC_CTOR(Consumer) { SC_THREAD(consume); } }; int sc_main(int argc, char* argv[]) { sc_fifo<int> fifo(10); Producer producer("Producer"); Consumer consumer("Consumer"); producer.out(fifo); consumer.in(fifo); sc_start(6, SC_SEC); return 0; }

OUTPUT

RESULT

Thus the Producer-Consumer problem was successfully implemented using SystemC.