Exp No : 17

UART INTERFACING USING INTERRUPT IN KEIL

AIM : To receive serial data using interrupt.

SOFTWARE REQUIRED :

- PC with Windows OS

- Keil μVision IDE

- 8051 Simulator (Keil built-in)

- Serial Window (UART #1)

PROGRAM :

PROGRAM:

#include <reg51.h>

void serial_ISR(void) interrupt 4

{

if(RI)

{

P1 = SBUF;

RI = 0;

}

}

void main()

{

TMOD=0x20;

TH1=0xFD;

SCON=0x50;

TR1=1;

ES=1;

EA=1;

while(1);

}

Circuit Diagram:

Before interruption:

After interruption:

RESULT :

Thus, the serial interrupt using UART was successfully implemented and verified in Keil simulator. The interrupt was triggered upon keyboard input and serviced correctly using an Interrupt Service Routine.