Experiment 1: Flashing of LED Using AT89C51 Microcontroller Using Proteus

Aim

To write an assembly language program to flash the LED using the AT89C51 microcontroller.

Algorithm

- Start the program.

- Initialize the program memory from address 0000H.

- Set Port 2.0 high to turn ON the LED.

- Call the delay subroutine.

- Clear Port 2.0 to turn OFF the LED.

- Call the delay subroutine.

- Repeat the process continuously using an infinite loop.

- Stop.

Program

ORG 0000H

UP:     SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY:  MOV R4,#35

H1:     MOV R3,#255

H2:     DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT :

Result

Thus, the program has been successfully verified and executed.